NAME-

Module 10: Infrastructure Security and WAN Technologies

Sr. No. 13

Question: A Cisco Catalyst switch connects to what should be individual user PCs. Each port has the same port security configuration, configured as follows:

interface range gigabit Ethernet 0/1 - 24

switchport mode access

switchport port-security

switchport port-security mac-address sticky

Which of the following answers describe the result of the port security configuration created with these commands? (Choose two)

Answer: Option B and Option D

Sr. No. 14

Question: What is the Administrative Distance of internal EIGRP routes?

Answer: Option B – 90

Sr. No. 15

Question: When a subnet mask is presented in binary, what do the binary 1s represent?

Answer: Option A – The network portion of an associated address

Sr. No. 16

Question: Which switch would STP choose to become the root bridge in the selection process?

Answer: Option A – 32768: 11-22-33-44-55-66

Sr. No. 17

Question: Which of the following devices is used by the service provider to provide WAN services?

Answer: Option C – WAN switch

Sr. No. 18

Question: Your Cisco IOS router is acting as a DHCP server. Which command will display the addresses that have been handed out to clients on the LAN?

Answer: Option D – show ip dhcp bindings

Sr. No. 19

Question: Which of the following commands would you use to enable EIGRP only on those interfaces with an IP address from 10.1.1.0 through 10.1.1.63?

Answer: Option A – network 10.1.1.0 0.0.0.63

Sr. No. 20

Question: R3 has a static route configured that points toward the service provider. What command could you use to have R3 advertise an OSPFv3 default route to the internal network, regardless of whether R3 had its default static route?

Answer: Option E – default-information originate always in OSPFv3 router configuration mode

Sr. No. 21

Question: You are configuring dynamic NAT on your Cisco IOS router. Which command is used to verify the interfaces that are being used as the outside interface and the inside interface?

Answer: Option F – show ip nat statistics

Sr. No. 22

Question: When using the “show EtherChannel summary” command, what does the “u” flag signify?

Answer: Option C – In use

Sr. No. 23

Question: Which command could you enter to encrypt passwords?

Answer: Option D – All of the above

Sr. No. 24

Question: You are using a Cisco IOS router as a DHCP server. Which command is used to identify the IPv4 addresses that will be in the DHCP pool?

Answer: Option A – network

Sr. No. 25

Question: Which of the following statements are true regarding the processing of ACLs that have been applied to router interfaces? (Choose two)

Answer: Option A and Option C

Sr. No. 26

Question: Imagine you configured OSPFv2 in a small lab network. Which of the following answers list a condition that could keep the routers in your lab from learning all the IPv4 routes? (Choose two)

Answer: Option A and Option C

Sr. No. 27

Question: Which statements describe neighbor discovery functionality in IPv6? (Choose two)

Answer: Option A and Option D

Sr. No. 28

Question: Which IPv6 prefix will the typical enterprise network receive from the service provider?

Answer: Option E – /48

Sr. No. 29

Question: How should a switch be configured so that it could be accessed remotely?

Answer: Option C – Configure a gateway for the switch

Sr. No. 30

Question: Refer to the exhibit. A network technician is asked to design a small network with redundancy. All hosts are in the same VLAN. What conclusions can be made about this design?

Answer: Option B – Spanning-tree will need to be used.