Assignment : - 7

NAME :- ROHIT PIYUSH JITENDRA BHAI.

Module -7: Network fundamental --

Section 1: Multiple Choice

- Which of the following messages in the DHCP process are broadcasted? (Choose two)

- Request B. Offer C. Discover D. Acknowledge

- Ans:- C. Discover, D. Acknowledge

Explanation:

- The DHCP Discover message is broadcasted from the client to the server to locate a DHCP server.

- The DHCP Acknowledge message is broadcasted from the DHCP server to the client to confirm the lease of an IP address.

2- Which command would you use to ensure that an ACL does not block web-based TCP traffic?

A. permit any                                          B. permit tcp any any eq 80

C. permit tcp any eq 80                          D. permit any any eq tcp

- Ans:-

  - B. permit tcp any any eq 80

Explanation:

  - The command permit tcp any any eq 80 allows TCP traffic on port 80, which is used for HTTP (web traffic). This would ensure that web-based traffic is not blocked.

4o mini

3-Explain Network Topologies

Ans:- . Network Topologies

Network topology refers to the layout of network devices and cables. Common types include:

- Bus: Single central cable connecting devices.

- Star: Devices connected to a central hub/switch.

- Ring: Devices connected in a loop.

- Mesh: Devices interconnected in a web.

- Hybrid: Combination of different topologies.

4-Explain TCP/IP Networking Model

Ans:- The TCP/IP model consists of four layers:

- Application: Network services (e.g., HTTP, FTP).

- Transport: End-to-end communication (e.g., TCP, UDP).

- Internet: Routing and addressing (e.g., IP).

- Network Interface: Physical network transmission (e.g., Ethernet).

5-Explain LAN and WAN Network

Ans:-  LAN (Local Area Network): Small area, like a home or office, using high-speed connections.

WAN (Wide Area Network): Large geographical area, connecting multiple LANs (e.g., internet).

6-Explain Operation of Switch

Ans:- A switch operates by forwarding data frames based on MAC addresses. It learns the MAC addresses of connected devices and forwards data only to the destination device, reducing traffic.

7-Describe the purpose and functions of various network devices 7-Make list of the appropriate media, cables, ports, and connectors to

Ans:-   Router: Connects different networks, routes data between them.

Switch: Connects devices within a network, forwards data based on MAC addresses.

Hub: Basic device to connect devices in a network (less efficient than a switch).

Firewall: Monitors and filters incoming/outgoing traffic for security.

Modem: Converts digital data to analog for internet access (or vice versa).

8-connect switches to other

Ans:-

Cables: Cat5e, Cat6 Ethernet cables.

Ports: RJ45 ports for Ethernet.

Connectors: RJ45 connectors for wired connections.

9-Define Network devices and hosts

Ans:-

Network Devices: Devices that manage or facilitate network communication (e.g., routers, switches, firewalls).

Hosts: End devices that use the network (e.g., computers, printers, servers).