Assignment : - 9

NAME :- ROHIT PIYUSH JITENDRA BHAI.

Module -9: : Module 10: infrastructure Security an WAN Technologies

13- A Cisco Catalyst switch connects to what should be individual user PCs. Each port has the same port security configuration, configured as follows: interface range gigabit Ethernet 0/1 - 24

switchport mode access switchport port-security

switchport port-security mac-address sticky Which of the following answers describe the result of the port security configuration created with these commands? (Choose two)

A. Prevents unknown devices with unknown MAC addresses from sending data through the switch ports.

B. If a user connects a switch to the cable, prevents multiple devices from sending data through the port.

C. Will allow any one device to connect to each port, and will save that device’s MAC address into the startup-config

D. Will allow any one device to connect to each port, but will not save that device’s MAC address into the startup-config

Ans:- A and B are the best two correct answers based on what the question is asking for — the  results  of the configuration.

14- What is the Administrative Distance of internal EIGRP routes?

A. 170     B. 90    C. 20       D. 1      E.110       F. 120

Ans:- 90     Administrative Distance (AD) is used by routers to select the best path when there are multiple routing protocols.

EIGRP (Internal) has an AD of 90

EIGRP (External) has an AD of 170.

15- When a subnet mask is presented in binary, what do the binary 1s represent? A. The network portion of an associated address

B. The host portion of the subnet mask

C. The number of wildcard bits in the subnet mask

D. The number of wildcard bits in the address

E. The network portion of the subnet mask

F. The host portion of an associated address

Ans:- A. The network portion of an associated address

In a subnet mask:  1s represent the network portion.

0s represent the host portion.

16- Which switch would STP choose to become the root bridge in the selection       process?

A. 32768: 11-22-33-44-55-66        B. 32768: 22-33-44-55-66-77

C. 32769: 11-22-33-44-55-65        D. 32769: 22-33-44-55-66-78

Ans:-  32768: 11-22-33-44-55-66

STP (Spanning Tree Protocol) selects the root bridge based on the lowest bridge ID, which is composed of: Bridge Priority (default is 32768)

MAC Address

Among equal priorities, lowest MAC address wins.

17- Which of the following devices is used by the service provider to provide WAN services?

A. Router                                  B. Core router

C. WAN switch                        D. CSU/DSU

Ans:-  CSU/DSU

CSU/DSU (Channel Service Unit/Data Service Unit):

Connects a customer's router to the WAN provider's network.

It acts as a digital interface for T1/E1 and other WAN circuits.

18- Your Cisco IOS router is acting as a DHCP server. Which command will display the addresses that have been handed out to clients on the LAN?

A. show ip dhcp assignments               B. show ip dhcp address

C. show ip dhcp conflicts                     D. show ip dhcp bindings

E. show ip dhcp pool

Ans:- D. show ip dhcp binding

Explanation: This command shows the IP addresses that have been leased to clients.

19- Which of the following commands would you use to enable EIGRP only on         those interfaces with an IP address from 10.1.1.0 through 10.1.1.63?

A. network 10.1.1.0 0.0.0.63        B. network 10.1.1.0/63

C. router eigrp 10.1.1.0 0.0.0.63  D. network 10.0.0.0 0.0.0.255

Ans:- A. network 10.1.1.0 0.0.0.63

Explanation: The wildcard mask 0.0.0.63 matches the subnet range 10.1.1.0/26.

20-R3 has a static route configured that points toward the service provider. What command could you use to have R3 advertise an OSPFv3 default route to the internal network, regardless of whether R3 had its default static route?

A. The decision to advertise a default route depends on the static route always being present on R3.

B. The default behavior is to redistribute any default IPv6 routes into OSPFv3, so no action is required.

C. Each of the other routers needs a static default route that leads to R3.

D. Use the command default-information originate always in interface mode for G1/0 on R3.

E. Have R3 use the command default-information originate always in OSPFv3 router configuration mode.

Ans:- E. Have R3 use the command default-information originate always in OSPFv3 router configuration mode.

Explanation: The always keyword ensures the default route is advertised even if R3 doesn’t currently have one.

21- You are configuring dynamic NAT on your Cisco IOS router. Which command is used to verify the interfaces that are being used as the outside interface and the inside interface?

A. show interfaces                             B. show ip route

C. show ip nat translations               D. show ip interface brief

E. show ip interface                         F. show ip nat statistics

Ans:- E. show ip interface

Explanation: This command displays NAT configuration (inside/outside) on each interface.

22- When using the “show EtherChannel summary “command, what does the “u “flag signify?

A. Waiting to be aggregated            B. Suspended

C. In use                                            D. Unsuitable for bundling

Ans-C. In use

Explanation: The lowercase “u” indicates the port is up and bundled in the channel.

23- Which command could you enter to encrypt passwords?

A. enable secret                                    B. username {username} secret {password} C. service password-encryption           D. All of the above E. None of the above

Ans:- D. All of the above

Explanation:

enable secret encrypts the enable password .

username ... secret ... stores encrypted passwords.

service password-encryption encrypts plain-text passwords in the config.

24- You are setting up a Cisco IOS router as a DHCP server. Which command is used to identify the IPv4 addresses that will be in the DHCP pool?

A. network                                B. dns-server           C. default-router                       D. ip dhcp excluded-address    E. lease                     F. ip dhcp pool                         G. domain-name

Ans:- A. network

Explanation: This command defines the subnet from which DHCP will assign addresses

25- Which of the following statements are true regarding the processing of ACLs that have been applied to router interfaces? (Choose two)

A. Inbound ACLs will be processed before the routing table lookup occurs              B. Inbound ACLs will be processed after the routing table lookup has occurred    C. Outbound ACLs will be processed after the routing table lookup has occurred D. Outbound ACLs will be processed before the routing table lookup occurs

Ans:- A. Inbound ACLs will be processed before the routing table lookup occurs

C. Outbound ACLs will be processed after the routing table lookup has occurred

Explanation: ACLs behave differently based on direction. Inbound is checked before routing, outbound is after.

26- imagine you configured OSPFv2 in a small lab network. Which of the following answers list a condition that could keep the routers in your lab from learning all the routes to all the IPv4 routes in your small lab network? (Choose two)

A. An ACL could be blocking router advertisements.                                                 B. Two neighbouring routers that connect to the same link have been configured with the same OSPF area and with the same IPv4 subnet mask.                             C. Any physical layer problem that would prevent two neighbouring routers from being able to ping each others IPv4 addresses in the subnet that exists between the two routers.                                                                                                          D. Two neighboring routers that connect to the same link have been configured with the same OSPF process ID on the router ospf command.

Ans:- A. An ACL could be blocking router advertisements.

C. Any physical layer problem that would prevent routers from pinging each other.

Explanation: ACLs and physical connectivity issues can prevent OSPF neighbors from forming.

27- Which statements describe neighbor discovery functionality in IPv6? (Choose two)

A. Determines the link layer address of a neighbor                                                       B. Finds neighbor switches on the link                                                                               C. Is achieved by using Dynamic Host Configuration Protocol for IPv6, or DHCPv6 with IPv6 multicast                                                                                                          D. Queries for duplicate addresses

Ans:- A. Determines the link layer address of a neighbor

D. Queries for duplicate addresses

Explanation: ND performs MAC discovery and DAD (Duplicate Address Detection).

28- Which IPv6 prefix will the typical enterprise network receive from the service provider?

A. /52   B./56   C. /64 D. /32 E. /48 F. /60

Ans:- E. /48

Explanation: Enterprises typically receive a /48, allowing them to subnet internally.

29- How should be configured a switch so that it could be accessed remotely?

A. Assign a password and privilege level

B. Apply the access control list, or ACL, to the virtual type terminal, or vty, lines C. Configure a gateway for the switch                                                                                      D. Generate a certificate

Ans:- C. Configure a gateway for the switch

Explanation: A switch must have a default gateway to be accessed from outside its local subnet.

30-Refer to the exhibit. A network technician is asked to design a small network with redundancy. The exhibit represents this design, with all hosts configured in the same VLAN. What conclusions can be made about this design?

- This design will function as intended.                                                                   B. Spanning-tree will need to be used.                                                                                     C.The router will not accept the addressing scheme.                                                        D. The connection between switches should be a trunk.                                                          E. The router interfaces must be encapsulated with the 802.1Q protocol.

Ans:- B. Spanning-tree will need to be used.

Explanation: Redundant Layer 2 paths exist; STP is needed to prevent loops.